| Layer Files (V7A prefix) |  |
| --- | --- |
| L1/TopSignal/Component | CMP |
| L2/TopGround | LY2 |
| L3/Power Plane 1 | L3 |
| L14/Power Plane 2 | L14 |
| L15/BottomGround | L15 |
| L16/BottomSignal/Solder | SOL |
| Silk Screen CMP | PLC |
| Silk Screen SOL | PLS |
| Solder Stop Mask CMP | STC |
| Solder Stop Mask SOL | STS |
| Cream Frame CMP | CRC |
| Cream Frame SOL | CRS |
| Excellen Drill Data | DDD |

PCB DRAWING

Subsystem: MCAT | ASSY: MCAT-CANYVAL | P/N: TOP | REV: 5X | 201502241100

By Samudra Haque (samudra@gwu.edu) and George Teel (gteel@gwu.edu), George Washington University, 2/23/2015

X-ray image of PCB displayed on next page.

Solder Jumper at position 3V3 position (1-2) of JMP1-0R. Leave 2&3 position  vacant.

Notes (unless otherwise specified)

- Fab per IPC-600G Class 2

- Material

  - Lead Free Assembly Compliant, ARLON 85NT or equiv.

  - Finished Thickness to be 0.062” +/- .005”

  - Total of 6 layers with 2 oz. CU ON ALL LAYERS.

- Size: Cut and mill to outline shown; . Datum is BoTTOM RIGHT  CORNER W/SHARP EDGE.

- DRILLING: DRILL HOLES PER SCHED. PLATE THROUGH HOLES WITH COPPER, 0.001” THICK MIN.

- MILLING: PCB OUTLINE PROVIDED

- ALL HOLE SIZES SPEC. AFTER PLATING.

- HOLE LOCATION ToL. ARE +/- .003” IN REL. To CTR.

- FINISH:

  - HASL

  - SOLDER MASK OVER BARE COPPER (SMOBC) USING Liquid PHOTO IMAGEABLE (LPI)

  - COLOR: BOTH SIDES, COLOR  BLUE,

  - SILKSCREEN:  WHITE.

- GERBER LAYERS MAY BE NAMED DIFFERENT, BUT LAYERS 1,2,3,14,15,16 (top>BOTTOM)

- LOWER EDGE HAS CONNECTORS UPTO BOARD EDGE or VERY CLOSE, CRITICAL POSITION.

Layer functions

L1/Top Signal/Top – 2 Oz Cu.

L2/Top Ground – 2 Oz Cu.

L3/Power Plane 1 – 2 Oz Cu.

L14/Power Plane 2 – 2 Oz Cu.

L15/Bottom Ground – 2 Oz Cu.

L16/Bottom Signal/Bottom – 2 Oz Cu.